#### HD/WS Technical Specification Requirements

#### APT EXCHANGE PROGRAMS
(February 1, 2011)

Master Tape Format

HDCam NTSC

Widescreen (WS): Digital Beta

Video Format

HD: (1080i)

WS: (480i)

Program Length

(HH:MM:SS) Program total running time (TRT) If your program does not meet these

requirements please contact us.

(1/30)    00:26:46

(1/60)    00:56:46

(1/90)    01:26:46

(1/120)  01:56:46

Children’s programming MUST be:

(1/30)    00:28:46

(1/60)    00:58:46

NOTE: TRT is from SOM (start of message, the first frame of program content) to EOM (end of message, the last frame of program content).  TRT is inclusive of ALL video elements through the ending :03 animated APT distributor logo.

Multiple Reels

Transition from reel to reel should not contain overlap.  Reels should follow with a next frame transition from reel one.  Subsequent reels must be slated/labeled, The first video on all reels must start at 01:00:00:00. Transitions should occur at black, a natural break, or a shot change held for +/- one (1) frame.

Aspect Ratio

Programs will be produced in 16:9 aspect ratio.  To protect all titles and credits during down conversions, the program must be 4:3 title, content, and center cut safe.

Evaluation

All tapes, including promos, submitted for broadcast must be delivered with a fully completed APT technical evaluation form available at:   APTonline.org/producers/distributionservices    Evaluations should be done after the captioning process. Tapes delivered without eval forms will not be processed and are subject to late fees.

Program Format

:60 seconds of NTSC color bars, with audio test tone

Starting with the test signal, time code must be SMPTE drop frame. The recording 	must begin with timecode starting at 00;58;30;00, with 01;00;00;00 being the SOM.

The time code must increment without interruption until 30 seconds after the :03

animated APT distributor logo.

The field rate for high definition is 59.94 fields per second.

The frame rate for high definition is 29.97 frames per second

VITC and LTC time code must be continuous and identical

:20 second visual slate containing

Title:       Your APT Program

Episode: Title (if applicable)

NOLA:    Four (4) letter code. Provided by APT

Eps. #:   One off use: 000, Series use episodic numbering provided by APT

Version:  Provided by APT

Revision: 001 or revision # provided by APT

Closed captioning (CC)

Described Video (DVI), and 2nd audio programming (SAP) if included on master

Track audio information

Total Run Time (TRT)

Date of edit

NOTE: (On-screen title, episode title and press material must match each other exactly)

:10 Countdown clock requirements

The countdown clock must be a :10 countdown to :02, ending with 1.8 seconds (59 frames) of silent black between the countdown clock and the beginning of the program. Countdown clock may be supered over the slate, or stand alone in the :10 following the :20 slate.

NOTE:  Pledge Program Slating: After color bars and tone, pledge programs must have a

main slate that contains all the information as noted above, including the TRT of

the ENTIRE program, and each upcoming segment within the program.  A

countdown clock should follow prior to program content start.  As well, each

individual segment must be preceded by a slate noting all the information above

including the TRT of the segment itself, and a countdown clock.

Pledge Version format for breaks between segments must be: 
:10 black, :15 slate, :10 countdown to :02, ending with 1.8 black = :35

Optional top of show tease

Ratings

Ratings Icon   The appropriate TV ratings icon must be inserted in the upper left corner 		for the first 15 seconds of program material (either on the tease if at least :15 or in the 		body of the program). The icon should be 154 horizontal x 138 vertical pixels and have 		at least a 70% density. Either a white background with black text or a black 			background with white text may be used to ensure that the icon can be clearly seen 		against any program opening.  Available at:

APTonline.org/producers/distributionservices

E/I Bug   Placement of an E/I bug on all APT core mission children’s programs is

required if noted in your deal memo.  E/I bug must be present from SOM to EOM, have upper right positioning and be 4:3 title safe. A Targa file of the e/i bug with an accompanying alpha channel is available at:  APTonline.org/producers/distributionservices    This file has logo opacity, upper right positioning and sizing embedded.  The resolution of the file is 720x486.  The transparency level of the "E/I" bug is 85%.

V-Chip   Ratings information is not required.  If inserted, V-Chip

information should be inserted on Field 2 on Line 9.  Insertion must be in accordance

with SMPTE Standards.

Title/Open

Underwriting credit pod (:60 max)  All pods MUST be pre-approved by APT.

NOTE:  The underwriter credit is the only material that may occur prior to

program material.

Program Body (Including production credits & optional :05 lower third web markers)

Web markers:	Max 3 (1/30), Max 5 (1/60).  Must be pre-approved by APT.

NOTE:  Deleted word(s) must be silenced and not “bleeped”.  See closed

captioning specs for further information.

NOTE:  During Pledge Event breaks the lower third must be graphics free

to avoid covering station inserted contact information

Closing Elements

NOTE:  The order of closing elements after program body and before the :03 animated APT distributor logo is at the discretion of the producer and may include the following:

:60 max underwriting credit pod (all pods MUST be pre-approved by APT)

:15 max web site announcement (MUST be pre-approved by APT)

:30 max product offer (offer may air before or after production credits).

(MUST be pre-approved by APT)

:05 maximum Producer or Presenter credit/logo

:05 maximum Producer or Presenter credit/logo

:01 black (required)

:03 animated APT distributor logo (required and must be the final element in

packaging.)  Available at:  APTonline.org/producers/distributionservices

:30 of black and silence following the :03 animated APT distributor logo. (required)

NOTE: A program format sample may be found at:

APTonline.org/producers/distributionservices

Closed Captioning

All content must be captioned; from SOM to EOM.  EIA 708 captions with EIA 608 compatibility bytes must be encoded on line 9 per SMPTE 334M.  Either Pop on or roll up is acceptable, DVI (Descriptive Video) is optional. When letterboxing, caption line must be completely masked.

Please note that deleted word(s) must be silenced and not “bleeped”. The captioning must match the audio exactly. Caption silenced words as follows:

Example:  	“The shit hit the fan.”

Audio: 	“The (silence) hit the fan”.

Caption: The (…) hit the fan.

Note: Captions during breaks in Pledge Event programs must be at the upper third position (to avoid covering station inserted contact information)

Audio

Reference tone

The operating level for reference tone and legacy analog system calibration is -20 dBFS per

SMPTE RP155.

Loudness level

Programs are to have average loudness levels that fall between -28 dBFS and -20 dBFS

during the majority of a program as measured on a digital meter calibrated to the RMS/VU ballistic. Average loudness should not go above -17 dBFS at any point during the program.

Peak audio

Programs are permitted to have audio levels that regularly peak near but not above

-3 dBFS using a digital peak meter.

Dialog level

Programs must be mixed using dialog level LAeq metering or a subsequent ATSC standard

method. They must have dialog levels with a value of -24 dBFS (+/-) 2 dB.  During moments

of dramatic impact, programs may have peak music or effects levels up to -3 dBFS as long as dialog levels are maintained as specified in this section.

Audio Track Assignment

Audio Phasing & Synchronization

Audio timing and phase must remain consistent across all channels and tracks.

Audio/video synchronization (lip sync) must appear to be correct during the program itself. During the program countdown, or by any in-service measurement technique adopted in the future, audio cannot lead or lag video by more than one half frame.

Note: Programs must have audio on all 4 tracks

WS Stereo Audio

Track 1: Left channel stereo or Mono

Track 2: Right channel stereo or Mono

Track 3: DVI or Mono mix (M & E mix unacceptable)

Track 4: Secondary Language – SAP, or Mono mix (M & E mix unacceptable)

HD Stereo Audio

Track 1: Left channel stereo or Mono

Track 2: Right channel stereo or Mono

Track 3: DVI or Mono mix (M & E mix unacceptable)

Track 4: Secondary Language – SAP, or Mono mix (M & E mix unacceptable)

Dolby E 5.1 Surround Configuration
		Channel/Track 1 - 4 Assignments

1 Left Stereo, Mono, or Left Total

2 Right Stereo, Mono, or Right Total

3 Dolby E (see next table)

4 Dolby E (see next table)

Channel/Track 3 and 4 Dolby E Assignments

Ch 1 Left Front

Ch 2 Right Front

Ch 3 Center

Ch 4 Low Frequency Effects

Ch 5 Left Surround

Ch 6 Right Surround

Ch 7 DVI (Mono) or Mono program (M & E mix unacceptable)

Ch 8 SAP (Mono) or Mono program (M & E mix unacceptable)

The producer must insure that the encoded Dolby signal is recorded on tape one frame

advanced from the recorded video. This is accomplished by delaying video and audio Tracks 1

and 2 by two frames. The delay shall be accomplished with methods external to the VTR.

Thus, all audio signals will be in sync with video when played back through a normal Dolby E decoder on an HDCam VTR in its default configuration.

Video

Video Level

Video levels must be measured with direct digital waveform monitoring equipment calibrated to

represent video levels on the final submission.

The luminance level in black areas of the program must fall as low as 0%, but not below 0%. Objectionable black clipping must not be evident. Black setup is not allowed in any digital submissions. When measured as derived composite signal, the black level must not fall below 7.5 IRE units.

The luminance level in white areas of the program must reach as high as 100%, but not above 100%. Objectionable white clipping must not be evident. When measured as derived composite signal, the white level must not exceed 100 IRE units.

Derived standard definition NTSC peak chrominance plus luminance gamut must not exceed 120 IRE when measured with a flat filter on a digital waveform monitor. Component digital derived GBR values must remain within the legal gamut of 0-700 millivolts.

Color Gamut

The allowable color gamut (range of values) for GBR signals in NTSC is 0-700 millivolts for Green, Blue, and Red. Serial digital 601 signals provide a Y Cb Cr component signal that can contain invalid colors when converted to GBR. Signals outside the GBR gamut range may lead to clipping, crosstalk or other distractions in encoders and other processing devices designed to operate within the legal gamut.

All G,B or R signals should lie inside the range -10 millivolts and 720 millivolts after an IRE filter has been applied. The resulting composite luminance signal should lie inside the range of -1 to 103 IRE. A diamond arrow or equivalent display should be used to detect illegal gamut levels.

Programs with GBR Upper Gamut levels above 735 millivolts; and GBR Lower Gamut below -50 millivolts will be rejected and returned to the program producer for correction.

GBR gamut should be “legalized” to the -10 to 720 millivolt range prior to submission to APT.

Labeling

Tape case and video tape must indicate the following:

NOTE: (On-screen title, episode title and press material must match exactly)

Master or Dub B

Title: Your APT Program

Episode Title (if applicable)

NOLA:    four (4) letter code. Provided by APT

Eps. #:   One offs should use 000, Series should use episodic numbering provided by APT

Version:  Provided by APT

Revision: 001 or revision # provided by APT

Closed captioning (CC)

Described Video (DVI), and 2nd audio programming (SAP) if included on master

Track audio information

Total Run Time (TRT)

Date of edit

Delivery

Master, Dub Bs, DVD screener and :30 Promo must be delivered no later than EIGHT weeks prior to assigned feed date.  See deal memo for information specific to your program(s).

Tapes arriving late will be subject to late delivery processing fees.

The APT Late Delivery Processing Fee was revised as of January 1, 2009.  The below charges apply if the above delivery date or specified schedule is not met. Please note these charges apply per episode in a series delivery.

43 – 56 days prior to feed       $100.

29 – 42 days prior to feed       $200.

15 – 28 days prior to feed       $300.

8 - 14   days prior to feed        $500.

Less than 7 days prior to feed $1000.

Ship all tapes to:

Distribution Services

American Public Television

55 Summer Street 4th Floor

Boston, MA 02110

Tel: 617-338-4455

NOTE: Do not use fiber filled bags or peanuts as tapes will be damaged using these

materials.  Supplier will be responsible for replacement of tape(s).