#### SD Promo Technical Requirements

#### APT EXCHANGE PROGRAMS
(Revised June 2009)

Tape Format

For SD material: 1 BetaSP or D-Beta tape

Promos MUST be delivered on a tape separate from the program material.

All tapes must be 4:3 aspect ratio or letterboxed if source material is 16:9 anamorphic.

Content/Length

:30 generic promo is required for series and one offs

Additional :20 promos are optional. Series episodic promos will be fed on the promo reel if

schedule availability permits.

Promo open must indicate visual or audio mention of the title of the program

Last: 05 of promo MUST be lower third title graphic clear, and clear of voice over, providing a

: 05 tag bed for local tune in information.

Music and effect audio during the last: 05 must be at a level that suits adding a voice-over.

All promo timings must be frame accurate to: 30.

All spots must end with a cut to black.

Packaging

:60 seconds of NTSC color bars, with audio test tone

Starting with the test signal, time code must be SMPTE drop frame. The recording should

Begin with time code starting at 00;58;30;00, with 01;00;00;00 being the SOM.

The time code must increment without interruption until :30 after the final program

segment.

The field rate for standard definition is 59:94 fields per second.

The frame rate for standard definition is 29:97 frames per second.

VITC and LTC time code must be continuous and identical, recorded on lines 16 and 18

in NTSC.

;20 seconds visual slate containing

NOTE: (On-screen title, episode title, tape case and box labels and press material must

match exactly)

Title: Your APT Program

Episode Title (if applicable)

NOLA:    four (4) letter code. Provided by APT

Eps. #:   One off use: 000, Series use episodic numbering provided by APT

Version:  Provided by APT

Revision: 001 or revision # provided by APT

Closed captioning (CC)

Described Video(DVI), and 2nd audio programming (SAP) if included on master

Track audio information

Date of edit

:10 Countdown clock requirements

The countdown clock must be a :10 countdown to :02 ending with 1.8 seconds (59 frames) of silent black between the countdown clock and the beginning of the program. Countdown clock may be supered over the slate, or stand alone in the :10 following the

:20 slate.

Closed Captioning

Captioning is not currently mandatory on promos.

Multiple Cuts

Cuts must be separated by :03 of black

Additional cuts must be individually slated.

:10 countdown is mandatory for each additional cut.

Audio

Reference tone

The operating level for reference tone and legacy analog system calibration is -20 dBFS per

SMPTE RP155.

Loudness level

Programs are to have average loudness levels that fall between -28 dBFS and -20 dBFS

during the majority of a program as measured on a digital meter calibrated to the RMS/VU

ballistic. Average loudness should not go above -17 dBFS at any point during the program.

Peak audio

HD Programs are permitted to have audio levels that regularly peak near but not above

-3 dBFS using a digital peak meter.

Dialog level

Programs must be mixed using dialog level LAeq metering or a subsequent ATSC standard method. They must have dialog levels with a value of -24 dBFS ±2 dB.  During moments of dramatic impact, programs may have peak music or effects levels up to -3 dBFS as long as dialog levels are maintained as specified in this section.

Audio Track Assignment

Stereo Audio

Track 1: Left channel stereo or mono

Track 2: Right channel stereo or mono

Track 3: DVI or Mono mix

Track 4: Secondary Language – SAP, or Mono mix

Dolby Matrixed Surround Sound

Track 1: Left total channel stereo

Track 2: Right total channel stereo

Track 3: Mono Mix of tracks 1 and 2, primary SAP or DVS

Track 4: Secondary language (SAP) or mono

Video

Video Level

Video levels must be measured with direct digital waveform monitoring equipment calibrated to represent video levels on the final submission.

The luminance level in black areas of the program must fall as low as 0%, but not below 0%. Objectionable black clipping must not be evident. Black setup is not allowed in any digital submissions. When measured as derived composite signal, the black level must not fall below 7.5 IRE units.

The luminance level in white areas of the program must reach as high as 100%, but not above 100%. Objectionable white clipping must not be evident. When measured as derived composite signal, the white level must not exceed 100 IRE units.

Derived standard definition NTSC peak chrominance plus luminance gamut must not exceed 120 IRE when measured with a flat filter on a digital waveform monitor. Component digital derived GBR values must remain within the legal gamut of 0-700 millivolts.

Color Gamut

Color Gamut

The allowable color gamut (range of values) for GBR signals in NTSC is 0-700 millivolts for Green, Blue, and Red. Serial digital 601 signals provide an Y Cb Cr component signal that can contain invalid colors when it is converted to GBR. Signals outside the GBR gamut range may lead to clipping, crosstalk or other distractions in encoders and other processing devices designed to operate within the legal gamut.

All G,B or R signals should lie inside the range -10 millivolts and 720 millivolts after an IRE filter has been applied. The resulting composite luminance signal should lie inside the range of -1 to 103 IRE. A diamond arrow or equivalent display should be used to detect illegal gamut levels.

Programs with GBR Upper Gamut levels above 735 millivolts; and GBR Lower Gamut below -50 millivolts will be rejected and returned to the program producer for correction.

GBR gamut should be “legalized” to the -10 to 720 millivolt range prior to submission to APT.

Labeling

On-screen title, episode title, tape case and box labels and press material must match exactly.

(See packaging information above for details.)

Delivery

:30 Promo must be delivered no later than EIGHT weeks prior to feed date.

All promo materials to be shipped to:

Distribution Services - PROMO

American Public Television

55 Summer Street

4th Floor

Boston, MA 02110

NOTE: Do not use fiber filled bags or peanuts as tapes will be damaged using these

materials.  Supplier will be responsible for replacement of tape(s).