#### SD Technical Specification Requirements

#### APT SYNDICATION PROGRAMS

#### (Revised February 1, 2011)

Tape Format

Per program:

Master - one (1) D-Beta or BetaSP

Screener - one (1) DVD (if series 1 per episode) if not previously provided.

Program Length

(HH:MM:SS) Preferred program total running time (TRT).

(1/30)    00:26:46

(1/60)    00:56:46

(1/90)    01:26:46

(1/120)  01:56:46

Multiple Reels

Transition from reel to reel should not contain overlap.  Reels should follow with a next frame transition from reel one.  Subsequent reels must be slated/labeled, The first video on all reels must start at 01:00:00:00. Transitions should occur at black, a natural break, or a shot change held for +/- one (1) frame.

Aspect Ratio

Aspect ratio must be 4:3, either full screen OR letterboxed if native format is HD or Widescreen/Anamorphic.  All content must be title and content safe.

Except for special effect purposes, the letterbox color must be black, and the IRE must

not fall below 7.5.

Program Format

:60 seconds of NTSC color bars, with audio test tone

Starting with the test signal, time code must be SMPTE drop frame. The recording 	must begin with timecode starting at 00;58;30;00, with 01;00;00;00 being the SOM.

The time code must increment without interruption until 30 seconds after the final audio/video.

The field rate for standard definition is 59:94 fields per second.

The frame rate for standard definition is 29:97 frames per second

VITC and LTC time code must be continuous and identical, recorded on lines 16

and 18 in NTSC.

:20 second visual slate containing

Title:     Your APT Program

Episode: Title (if applicable)

Eps. #:  (if applicable)

Closed captioning (CC)

Described Video (DVI), and 2nd audio programming (SAP) if included on master

Track audio information

Total Run Time (TRT)

NOTE: (On-screen title, episode title and press material must match each other exactly)

:10 Countdown clock requirements

The countdown clock must be a :10 countdown to :02, ending with 1.8 seconds (59 frames) of silent black between the countdown clock and the beginning of the program. Countdown clock may be supered over the slate, or stand alone in the :10 following the :20 slate.

NOTE:  Pledge Program Slating: After color bars and tone, pledge programs must have a

main slate that contains all the information as noted above, including the TRT of

the ENTIRE program, and each upcoming segment within the program.  A

countdown clock should follow prior to program content start.  As well, each

individual segment must be preceded by a slate noting all the information above

including the TRT of the segment itself, and a countdown clock.

Pledge Version format for breaks between segments must be: 
:10 black, :15 slate, :10 countdown to :02, ending with 1.8 black = :35

Optional top of show tease

Title/Open

Underwriting credit pod (:60 max)  All pods MUST be pre-approved by APT.

NOTE:  The underwriter credit is the only material that may occur prior to

program material.

Program Body (Including production credits & optional :05 lower third web markers)

Web markers:	Max 3 (1/30), Max 5 (1/60).  Must be pre-approved by APT.

NOTE:  Deleted word(s) must be silenced and not “bleeped”.

(See closed captioning specs for further information)

NOTE:  During Pledge Event breaks the lower third must be graphics free

to avoid covering station inserted contact information.

Closing Elements

NOTE:  The order of closing elements after program body and before the :03 animated APT distributor logo is at the discretion of the producer and may include the following:

:60 max underwriting credit pod (all pods MUST be pre-approved by APT)

:15 max web site announcement (MUST be pre-approved by APT)

:30 max product offer (offer may air before or after production credits).

(MUST be pre-approved by APT)

:05 maximum Producer credit/logo

:01 black (required)

:30 of black and silence following the :03 animated APT distributor logo. (required)

NOTE: A program format sample may be found at:

APTonline.org/producers/distributionservices

Closed Captioning

If a captioned Master is provided all content must be captioned; from SOM to EOM. The captioning data stream must be located

on line 21, field 1 in the vertical interval.  The level of the datastream pulses shall be 50 IRE

with a tolerance of (+/-) 5%.  In no case should the data stream extend below the baseline.

When letterboxing, caption line must be completely masked.

Please note that deleted word(s) must be silenced and not “bleeped”. The captioning must match the audio exactly. Caption silenced words as follows:

Example:  	“The shit hit the fan.”

Audio: 	“The (silence) hit the fan”.

Caption: The (…) hit the fan.

Audio

Reference tone

The operating level for reference tone and legacy analog system calibration is -20

dBFS per SMPTE RP155.

Loudness level

Programs are to have average loudness levels that fall between -28 dBFS and -20

dBFS during the majority of a program as measured on a digital meter calibrated to the RMS/VU ballistic. Average loudness should not go above -17 dBFS at any point during the 	program.

Peak audio

Programs are permitted to have audio levels that regularly peak near but not above

-10 dBFS using a digital peak meter.

Dialog level

Programs must be mixed using dialog level LAeq metering or a subsequent ATSC  standard method. They must have dialog levels with a value of -24 dBFS (+/-) 2 dB.  During moments of dramatic impact, programs may have peak music or effects levels up to -3 dBFS as long as dialog levels are maintained as specified in this section.

Audio Track Assignment

Audio Phasing & Synchronization

Audio timing and phase must remain consistent across all channels and tracks

Audio/video synchronization (lip sync) must appear to be correct during the program

itself. During the program countdown, or by any in-service measurement technique

adopted in the future, audio cannot lead or lag video by more than one half frame.

Stereo Audio

Track 1: Left channel stereo or mono

Track 2: Right channel stereo or mono

Track 3: DVI or Mono mix

Track 4: Secondary Language (SAP), or Mono mix

Dolby Matrixed Surround Sound

Track 1: Left total channel stereo

Track 2: Right total channel stereo

Track 3: Mono Mix of tracks 1 and 2, primary SAP or DVI

Track 4: Secondary language (SAP) or mono

The producer must insure that the encoded Dolby signal is recorded on tape one frame

advanced from the recorded video. This is accomplished by delaying video and audio Tracks 1

and 2 by two frames. The delay shall be accomplished with methods external to the VTR.

Thus, all audio signals will be in sync with video when played back through a normal Dolby E decoder on an HDCam VTR in its default configuration.

Video

Video Level

Video levels must be measured with direct digital waveform monitoring equipment calibrated to represent video levels on the final submission.

The luminance level in black areas of the program must fall as low as 0%, but not below 0%. Objectionable black clipping must not be evident. Black setup is not allowed in any digital submissions. When measured as derived composite signal, the black level must not fall below 7.5 IRE units.

The luminance level in white areas of the program must reach as high as 100%, but not above 100%. Objectionable white clipping must not be evident. When measured as derived composite signal, the white level must not exceed 100 IRE units.

Derived standard definition NTSC peak chrominance plus luminance gamut must not exceed 120 IRE when measured with a flat filter on a digital waveform monitor. Component digital derived GBR values must remain within the legal gamut of 0-700 millivolts.

Color Gamut

Color Gamut

The allowable color gamut (range of values) for GBR signals in NTSC is 0-700 millivolts for Green, Blue, and Red. Serial digital 601 signals provide a Y Cb Cr component signal that can contain invalid colors when converted to GBR. Signals outside the GBR gamut range may lead to clipping, crosstalk or other distractions in encoders and other processing devices designed to operate within the legal gamut.

All G,B or R signals should lie inside the range -10 millivolts and 720 millivolts after an IRE filter has been applied. The resulting composite luminance signal should lie inside the range of -1 to 103 IRE. A diamond arrow or equivalent display should be used to detect illegal gamut levels.

Programs with GBR upper gamut levels above 735 millivolts; and GBR lower gamut below -50 millivolts will be rejected and returned to the program producer for correction.

GBR gamut should be “legalized” to the -10 to 720 millivolt range prior to submission to APT.

Delivery

Master, DVD screener and must be delivered no later than EIGHT weeks prior to start of contract.

Ship SD Master and DVD screener to:

Distribution Services

American Public Television

55 Summer Street

4th Floor

Boston, MA 02110

Tel: 617-338-4455

NOTE: Do not use fiber filled bags or peanuts as tapes will be damaged using these materials.  Supplier will be responsible for replacement of tape(s).